Using online resources, to come up with a theoretical equation that would allow me to find the fundamental frequency of the string in relation to the change in temperature

  - Theoretical relationship would begin with the formula for frequency is:

f= 12LTμ  Where f is frequency, L is length of the string that is clamped (between the two nodes), T is the tension of the string, and µ is the linear density of the string.

  - The formula for the linear density of the string can be written as:

mL where m is the mass of the string, and L is the length of the clamped string between my two nodes.

  - Equation for change in tension t can be written as ∆T= σEA∆t, where ΔT is the change in tension of the string, σ is the coefficient of thermal expansion, E is Young’s modulus, and Δt is the change in temperature.

  - We know that the change in frequency will be the initial frequency subtracted from the final frequency, written as: ∆f =ffinal- finitial

  - Formula for frequency is: f= 12LTμ  , therefore, frequency final – frequency initial (ΔFrequency) is: f= 12LTfμ- Tiμ

  - Plugging in the equation for change in tension, I can get:                                                 f= 12LTi +∆T μ- Tiμ

  - Assuming f0 to be the frequency at temperature t0 and tension Ti, and f0+Δf (new frequency) to be the frequency at t0+Δt (new temperature) and Ti+ΔT (new tension), the equation can be rewritten as:

Δf= 12LTi+(σEA∆t)µ- 12LTiµ

  - This equation can be simplified by factoring the 12L out, which would result in: ∆f= 12L Ti+(σEA∆t)μ- Tiμ